# Lab 01 - Modulation

# Part I: Scavenger Hunt

Find the hidden signal that is being broadcast within the range

## Instructions

- Connect to one of the remote radios listed below using SDR#.

- Scan through the signals, the signal uses a narrow bandwidth

- Listen to the audio through your speakers, you’ll find a few different transmissions. If at first you think you’re getting trolled by the Russians, you’re close, but the trolling isn’t the secret message itself.

- Take a screenshot of discovering the signal. What is the signal saying (audio)?

## Findings and Analysis:

- What is the secret message?

The secret message is "Mary had a little lamb, its fleece was white as snow."

- Include a screenshot showing that you’ve found it in SDR#

Figure 1: Secret Message Signal

# Part II: Triangulation

Estimate the rough location of a mobile device given three base stations and key parameters

such as received power, distance, and antenna gains.

## Instructions

We know the basics for the relationship between received power, distance, and antenna gains. Given this information, we should be able to determine a mobile device’s rough location. In our scenario, we’re assuming we have free space propagation and that in our mystical world, all antennas’ gains are 1. Given the information below, determine the approximate location of the mobile device. Make sure to show/explain/demonstrate how you determined the location, and show what you’ve found on some sort of a map. How you choose to render the map is up to you.

## Findings and Analysis:

### Setup

#### Friis Formula

#### Pr=GrGtPt4πdλ2

| Pr | Received power the strength of the signal as it arrives at the receiving antenna |
| --- | --- |
| Gr | Gain of receiving antenna how effectively the receiving antenna captures the incoming signal |
| Gt | Gain of transmitting antenna. how effectively the transmitting antenna sends the signal in a particular direction |
| Pt | Transmitted power (watts) the power with which the signal is initially transmitted from the transmitting antenna. |
| d | Distance between transmitting and receiving antennas units for wavelength and distance must be the same |
| λ | Wavelength of the signal. λ = c / f, speed of light (c) and the frequency (f): units for wavelength and distance must be the same |

#### Find distance formula

Transform the Friis formula to a distance formula to directly calculate the approximate distance (d) between a mobile device and each base station.

| #### Pr=GrGtPt4πdλ2 | formula |
| --- | --- |
| #### Pr=GrGtPtλ24πd2 | fraction rule abc=a×cb |
| #### 4πd2×Pr=GrGtPtλ2 | multiply both sides by 4πd2 |
| #### 4πd2=GrGtPtλ2Pr | divide both sides by Pr |
| #### 4πd=GrGtPtλ2Pr | take square root of both sides |
| #### d=14π×GrGtPtλ2Pr | fraction rule if a×b×c=d, then c=da×b |

#### Calculate Wavelength

Given that all frequencies are 450 kHz, the wavelength (λ) can be determined before proceeding

| #### λ=cf #### c=speed of light f=frequency | formula |
| --- | --- |
| #### f=450 kHz #### = 450000 Hz #### = 4.5 × 105 Hz | convert frequency to Hz |
| #### c=3.0×108 m/s | notate speed of light |
| #### λ=3.0×108 4.5 × 105 | input values |

#### Calculate Distances

Perform calculations to determine distance for each specific base station using given formulas and information

##### Base Station 1:

###### Information

| Location | 44.012320, -97.109509 |
| --- | --- |
| Power Transmitted | 200 w |
| Power Received | 17.3512367 w |
| Transmitter Gain | 10 |
| Receiver Gain | 25 |
| Frequency | 450 kHz (4.5 × 105 Hz) |

###### Calculate Distance

| #### d=14π×GrGtPtλ2Pr | distance formula |
| --- | --- |
| #### d=14π×25×10 ×200 ×3.0×108 4.5 × 105217.3512367 | input known value |
| #### d=14π×25×10×200 ×444444.4417.3512367 | Solve for λ2 |
| #### d=14π×2222222200017.3512367 | multiplication |
| #### d=14π×1.2807284 × 109 | division |
| #### d=14π×35787.27 | solve for square root |
| #### d=2847.860m | multiplication |

###### Map

Figure 2: Base 2 | 44.012320, -97.109509, 2847.860m

[view base one map]

##### Base Station 2:

###### Information

| Location | 44.013371, -97.289582 |
| --- | --- |
| Power Transmitted | 200 w |
| Power Received | 0.99757704 w |
| Transmitter Gain | 10 |
| Receiver Gain | 25 |
| Frequency | 450 kHz (4.5 × 105 Hz) |

###### Calculate Distance

| #### d=14π×GrGtPtλ2Pr | distance formula |
| --- | --- |
| #### d=14π×25×10 ×200 ×3.0×108 4.5 × 10520.99757704 | input known value |
| #### d=14π×25×10×200 ×444444.440.99757704 | Solve for λ2 |
| #### d=14π×222222220000.99757704 | multiplication |
| #### d=14π×2.2276196 × 1010 | division |
| #### d=14π×149252.12 | solve for square root |
| #### d=11877.106m | multiplication |

###### Map

Figure 3: Base 3 | 44.013371, -97.289582, 11877.106m

[view base two map]

##### Base Station 3:

###### Information

| Location | 44.119244, -97.215958 |
| --- | --- |
| Power Transmitted | 200 w |
| Power Received | 0.9337055 w |
| Transmitter Gain | 10 |
| Receiver Gain | 25 |
| Frequency | 450 kHz (4.5 × 105 Hz) |

###### Calculate Distance

| #### d=14π×GrGtPtλ2Pr | distance formula |
| --- | --- |
| #### d=14π×25×10 ×200 ×3.0×108 4.5 × 10520.9337055 | input known value |
| #### d=14π×25×10×200 ×444444.440.9337055 | Solve for λ2 |
| #### d=14π×222222220000.9337055 | multiplication |
| #### d=14π×2.3800033 × 1010 | division |
| #### d=14π×154272.59 | solve for square root |
| #### d=12276.623m | multiplication |

###### Map

Figure 4: Base 3 | 44.119244, -97.215958, 12276.623m

[view base three map]

#### Triangulate Mobile Device

Use calculated distances to triangulate location of mobile device

In the process of locating the mobile device, circles were overlayed on a map to represent the calculated distances from each individual base station. The convergence point, or where these circles intersect, reveals the approximate location of the mobile device. The screenshots below illustrate this process.

Figure 5: OpenStreetMap view of Convergence Point

The OpenStreetMap (OSM) view shown in Figure 5 displays overlapping circles, with the the convergence point outlined. When zooming in on this focal point, a cell tower icon can be seen, which confirms accurate triangulation.

Figure 6: Cell Tower Icon

[view on map]

By drawing a circle around this icon, as shown in Figure 6, the exact location is determined to be 44.022843, -97.141968, or 44°01'22.2" N  97°08'31.1"W

Lastly, by examining Google Satellite View (Figure 7) and Google Street View (Figure 8), the triangulation location is further validated. The Satellite View provides an aerial perspective of the tower, while Street View provides a view from ground level.

Figure 7: Satellite View

View on Map

Figure 8: Street View

View on Map